# Smart Voice Assistant

# Project Documentation

## Submitted By

| Name | ID |
| --- | --- |
| Safwan Ahmed Rayed | 24701066 |
| Sheikh Tawsif Bin Aftab | 24701026 |
| Syed Mehraz Monwar | 24701042 |
| Sanjida Mahi | 24701009 |

# Abstract

Smart Voice Assistant is an ESP32-S3 based desk assistant that combines embedded hardware, a graphical TFT interface, voice recording, local networking, and AI-powered speech interaction. The device records a user's spoken command through a microphone, sends the audio to a local Windows server, uses an Android-hosted OpenAI-compatible AI service for speech-to-text, response generation, and text-to-speech, then plays the generated voice response through a speaker.

The project demonstrates how a low-power microcontroller can be connected with external AI services by using a local bridge server. The ESP32-S3 handles the embedded interface, touch and button controls, Wi-Fi communication, audio recording, and playback. The PC server handles API communication and audio conversion. The Android phone runs the AI models through Tool Neuron.

# Objectives

Build a compact smart voice assistant using ESP32-S3.

Create an animated TFT display interface for clock, weather, system, and AI states.

Record short voice commands using a microphone module.

Send recorded audio from ESP32-S3 to a local server over Wi-Fi.

Use an Android AI service for speech-to-text, text generation, and text-to-speech.

Play generated audio responses through an amplifier and speaker.

Provide setup, wiring, server, and firmware documentation for future reproduction.

# System Overview

The project is divided into three main parts:

| Part | Description |
| --- | --- |
| ESP32-S3 Device | Handles display UI, input controls, recording, Wi-Fi, and speaker playback |
| Windows AI Server | Receives audio, calls AI APIs, converts audio, and returns responses |
| Android AI Provider | Runs Tool Neuron or another OpenAI-compatible AI/STT/TTS backend |

# Hardware Components

| Component | Purpose |
| --- | --- |
| ESP32-S3 | Main microcontroller |
| ST7789 240x240 TFT Display | Visual user interface |
| MAX4466 Microphone Module | Voice input |
| TTP223 Touch Sensor | Page navigation |
| Push Button | AI mode and recording control |
| HW-104 Audio Amplifier | Speaker amplification |
| XL6009 Booster Module | Amplifier power supply |
| Speaker | Voice output |
| LED | Status/testing indicator |
| Breadboard and Jumper Wires | Circuit connection |
| Windows PC | Runs the local bridge server |
| Android Phone | Runs Tool Neuron AI service |

# Important Wiring Summary

| Module | Pin/Terminal | Connection |
| --- | --- | --- |
| MAX4466 Microphone | OUT | ESP32-S3 GPIO1 |
| TTP223 Touch Sensor | OUT | ESP32-S3 GPIO5 |
| Push Button | Signal | ESP32-S3 GPIO6 |
| HW-104 Amplifier | R | ESP32-S3 GPIO4 |
| HW-104 Amplifier | G | Shared GND |
| HW-104 Amplifier | L | Disconnected |
| HW-104 Amplifier | R+ / R- | Speaker terminals |
| XL6009 Booster | OUT+ / OUT- | Amplifier power input |
| ST7789 TFT | DC | ESP32-S3 GPIO9 |
| ST7789 TFT | MOSI | ESP32-S3 GPIO11 |
| ST7789 TFT | SCK | ESP32-S3 GPIO12 |

A common ground is required between the ESP32-S3, microphone, touch sensor, amplifier, and booster module.

# Software Components

| Software | Purpose |
| --- | --- |
| Arduino IDE / Arduino CLI | Firmware build and upload |
| Arduino_GFX Library | ST7789 display control |
| ESP32 Arduino Core | Wi-Fi, web server, preferences, and hardware control |
| Python | Local server runtime |
| FastAPI | HTTP server framework |
| Uvicorn | ASGI server runner |
| Tool Neuron | Android AI/STT/TTS provider |
| OpenAI-compatible API | Standard API format for AI requests |

# User Interaction Flow

| Action | Result |
| --- | --- |
| Touch sensor tap | Cycles display pages |
| First button press | Enters AI mode |
| Second button press | Starts voice recording |
| Third button press | Stops recording and sends audio |
| Server response received | Device displays answer state and plays audio |

# Working Principle

The ESP32-S3 boots and loads saved Wi-Fi and server settings.

The TFT display shows the main interface, including clock, weather, system, and AI pages.

The user enters AI mode using the push button.

The device records audio through the MAX4466 microphone.

The ESP32-S3 sends the recorded audio to the local FastAPI server.

The server converts the audio into WAV format.

The server sends the WAV file to Tool Neuron for speech-to-text.

The transcript is sent to a chat model to generate a response.

The response text is sent to a text-to-speech model.

The generated speech is converted into ESP32-compatible 8 kHz mono WAV audio.

The ESP32-S3 downloads the audio file and plays it through the HW-104 amplifier and speaker.

# AI Server Architecture

ESP32-S3
  |
  | recorded audio
  v
Windows FastAPI Server
  |
  | STT / chat / TTS API calls
  v
Android Tool Neuron Server
  |
  | transcript, response text, generated speech
  v
Windows FastAPI Server
  |
  | response JSON + WAV audio URL
  v
ESP32-S3

# Display Interface

The display interface is designed around an animated sci-fi style UI. The main visual states include:

Clock display with bold dot-matrix style time.

Weather display with icon-style visual information.

System information page.

AI listening face with bouncing dots.

Sending/waiting animation while the server processes the voice request.

Speaking animation while the AI response is being played.

Setup screen for Wi-Fi and server configuration.

# Firmware

Firmware file:

AI Desk Assistant/AI Desk Assistant.ino

Main firmware responsibilities:

ST7789 display drawing.

Wi-Fi connection.

Setup access point and configuration page.

Touch and button input handling.

Microphone sampling.

Audio upload to the server.

WAV download and speaker playback.

Clock, weather, system, and AI page rendering.

# Server

Server file:

AI Server/server.py

Main server responsibilities:

Receive audio from the ESP32-S3.

Convert raw ESP32 audio to WAV.

Send audio to the STT model.

Send transcript to the chat model.

Send response to the TTS model.

Convert audio to ESP32-friendly playback format.

Serve generated audio files to the ESP32-S3.

# Configuration

The server uses a local .env file based on:

AI Server/.env.example

Important configuration values:

| Setting | Purpose |
| --- | --- |
| TOOL_NEURON_BASE_URL | Android AI server /v1 endpoint |
| TOOL_NEURON_API_KEY | Local API token if required |
| TOOL_NEURON_CHAT_MODEL | Chat model ID |
| TOOL_NEURON_STT_MODEL | Speech-to-text model ID |
| TOOL_NEURON_TTS_MODEL | Text-to-speech model ID |
| PUBLIC_BASE_URL | PC server URL reachable by ESP32-S3 |
| PORT | Local server port |

# Testing And Result

The project was tested in stages:

TFT display initialization and UI rendering.

Touch input page cycling.

Button-based AI mode and recording flow.

Microphone recording from ESP32-S3.

Audio upload from ESP32-S3 to the Windows server.

AI model listing and API connectivity from the phone server.

Speech-to-text request handling.

Chat response generation.

Text-to-speech audio file generation.

ESP32-S3 audio download and playback attempt through the amplifier.

The AI pipeline successfully received recorded audio, generated responses, and produced output audio files. Speaker playback depends on correct amplifier wiring and power stability.

# Challenges

Managing limited ESP32-S3 memory while rendering animated UI and handling audio buffers.

Reducing display flickering during AI animations.

Converting server-generated audio into a simple ESP32-friendly WAV format.

Coordinating three systems at once: ESP32-S3, Windows PC, and Android phone.

Debugging amplifier wiring and speaker output.

Keeping private API/token values separate from public project files.

# Limitations

Voice recording length is short to fit ESP32-S3 memory limits.

Response time depends on the Android phone model and selected AI model.

The PC server must be running during AI mode.

The phone AI service must stay active on the same network.

Audio quality depends on microphone calibration, amplifier wiring, and power supply noise.

# Future Improvements

Add better audio filtering and microphone gain calibration.

Improve speaker output quality with a cleaner DAC/I2S amplifier path.

Add a more polished weather page with animated icons.

Add persistent model selection through the setup page.

Add a web dashboard for configuring server and device settings.

Add automatic local network discovery for the PC server.

Add longer recording support using streaming or external memory.

# Repository Structure

AI Desk Assistant/
  AI Desk Assistant.ino

AI Server/
  server.py
  requirements.txt
  .env.example

Setup Guide/
  README.md
  Configure-Server.ps1
  Start-Server.ps1
  Stop-Server.ps1
  Test-AI.ps1
  Build-Firmware.ps1
  Flash-Firmware.ps1

Documentation/
  AI_Desk_Assistant_Submission_Document.docx
  Wiring.md
  Project Notes.md
  Troubleshooting.md
  Report/

# Conclusion

Smart Voice Assistant shows how an ESP32-S3 can be used as the embedded front end for a modern AI voice assistant. By offloading AI processing to a local server and Android phone, the device can provide speech interaction while still using affordable microcontroller hardware. The project combines embedded systems, IoT networking, audio processing, user interface design, and AI service integration into one working prototype.